- Industrial Socket & Plugs

Industrial sockets and plugs are heavy-duty connectors for safe power distribution in harsh environments in PVC, GRP, Marine Aluminium. They come in various standards (IEC 60309, NEMA) current ratings (≈16 A to several hundred Amp), and pole configurations for single- and three-phase systems. Choose by current/voltage, number of poles/earth, IP rating (up to IP67), contact material, cable size and certifications.

- Interlocked Socket in Plastic/GRP/Metal

Interlocked sockets (available in plastic, GRP or metal) are safety sockets that prevent live power being supplied unless a matching plug is fully inserted and the associated enclosure/door or isolator is correctly closed. They combine a mechanical/electrical interlock with the socket so power is only enabled when safe with circuit protection (MCB/RCD/fuse), reducing risk during maintenance or when accessing enclosures.

- High Current Metal Socket & Plug

High-current metal sockets and plugs (250–800 A) are aluminum-alloy, corrosion-proof, tempered connectors with IP66/67 sealing, built for heavy industrial and marine use. We offer high current capacity, robust mechanical coupling, screw/bolted terminations for large conductors, integrated earthing and lockout options, and are chosen based on conductor size, voltage and certifications.

- High Current Interlocked Socket & Plug

High-current interlocked sockets and plugs (250–800 A) are aluminum-alloy, corrosion-proof, tempered connectors with IP67 sealing that prevent power unless a matching plug and enclosure. They deliver high current capacity and durable mechanical coupling, use screw/bolted terminations for large conductors, include integrated earthing and lockout/interlock features for safe maintenance, and are suited to heavy industrial, marine and power-distribution applications—select by conductor size, voltage, interlock compatibility and certifications.

- Single Pole Cam Connectors

They are a type of electrical connector primarily used in high-current applications, often seen in welding, temporary power distribution, and entertainment lighting setups with a "cam" mechanism to create a secure, high-conductivity connection between two conductors. Designed to handle high amperage loads, often ranging from 100 amps to well over 1500 amps. Often color-coded to prevent misconnections.

- Enclosures and Junction Boxes

Enclosures and junction boxes are available in PVC, GRP, marine aluminium and stainless steel, with IP ratings such as IP44, IP56 and IP67 for varying dust/water protection. Fire-resistant options (E30, E60, E90) provide timed integrity for emergency systems, and units come in multiple sizes and shapes to suit surface, flush or bespoke installations. Choose material and IP/fire rating based on environment (corrosive, marine, indoor/outdoor) and required cable entry/termination arrangements.

- Cable Glands and Lugs

Cable glands and lugs are key termination accessories: glands (PG/metric, NPT, compression, EMC, armoured, ex-proof, waterproof to IP68) come in nylon, brass (nickel-plated), stainless steel (316), aluminium and silicone-sealed variants; lugs (crimp, solder, mechanical, tubular, pin/flag, battery) are usually copper (tinned), aluminium or nickel-plated copper with stainless-steel hardware. Select size and material by conductor type, current rating and environment.

- Pre assembled Plug & Socket Box

Pre-assembled plug & socket boxes are ready-to-install units combining enclosure, inlet/outlet sockets, circuit protection (MCB/RCD/fuse), isolator, wiring and cable entries. They come in various formats (single/double/multi-socket, industrial IEC/BS/CEE, weatherproof/IP65–IP67, or Ex-rated), and materials (PVC, GRP, marine-grade aluminium, stainless steel).

- Isolators & Disconnection Switch

Isolators and disconnection switches safely interrupt supply for maintenance or emergencies. They come fused or non-fused, in single-, double-, 3- or 4-pole, and as load-break or no-load types (rotary, knife, cam, DIN-rail). Housings range from thermoplastic, aluminium to steel with IP ratings to IP66/67; ratings go from a few amps to thousands. Select by voltage/current, fault capacity, visible isolation needs and applicable standards.

- Portable Safety Transformers

Portable safety transformers step voltages up or down to safe levels for temporary power and Isolation transformers transfer electrical energy between circuits while electrically isolating them to enhance safety and performance. They’re compact, mobile, include multiple outlets and protective devices (fuses/MCB/RCD), and reduce electrical hazards. Choose by kVA rating, single- or three-phase, socket types, and IP/environment rating.

- Custom Build Lighting Junction box

Custom-built lighting junction boxes are compact, durable enclosures that protect and organize lighting connections. Made from UV-stable thermoplastic, GRP, or steel, they include gland entries, numbered terminal blocks, earth/neutral bars, and space for DIN-rail components. With IP54–IP66 ratings, gasketed lids, clear labeling, and optional pre-wiring, they suit street, architectural, car park, event, and industrial lighting; choose by circuit count, conductor size, IP/IK rating, and standards compliance.

- Custom Junction Box with IP68

Custom IP68 enclosures offer full protection against water ingress and are typically used in applications like offshore oil and gas platforms, the Thames Flood Barrier, river and reservoir monitoring systems, camera housings, underwater lighting fixtures, and lighting junction boxes. They are manufactured in a range of materials including 316L stainless steel, aluminium, GRP, and rigid polyurethane.

- Cable Tray And Accessories

A versatile range of cable management products designed for industrial, commercial, and residential installations, including perforated and ladder cable trays (light, medium, and heavy-duty), solid-bottom trays, wire-mesh trays, trunking, and a complete set of accessories such as bends, tees, reducers, couplers, splice plates, covers, support brackets, expansion joints, and cable dropouts. Available in Hot-dip galvanized and other protective finishes such as Power Coating for offshore Installation, these products offer high tensile strength, excellent corrosion resistance, configurable load capacities, and options for custom lengths and configurations. Cable trays are commonly made from Steel: Often galvanized for corrosion resistance, Stainless Steel (SS316L), Aluminum: Lightweight and corrosion-resistant, Glass/Fiber-Reinforced Plastic (GRP/FRP): For highly corrosive environments.

Certifications

Nill